AMERICOLD MANUFACTURING PROCESS SHEET

| PROCESS DESCRIPTION: MACHINING ALUMINUM CONNECTING RODS | PS #: 2104-003 |
| --- | --- |
| DEPARTMENT: 2104 | REV.: 1 |
| PART NAME: ALUMINUM CONNECTING ROD | DATE: |
| PART NO.: 312-333, -334, -344 | PREPARED BY: P. Brandon |
| MACHINE NAME: WADELL CONNECTING ROD BORE MACHINE | APPROVED BY: |
| MACHINE NUMBER: 1-0271, 1-0288 | PAGE 1 OF 1 |

- Finish Bore Crankline & Wristpin Holes.

TOOLS REQUIRED

|  | CRANKLINE BORE |  |  |
| --- | --- | --- | --- |
|  | TOOL NUMBER | DESCRIPTION |  |
|  | C60-13K39 | Boring Bar |  |
|  | C60-13K36A | Boring Tool Rough |  |
|  | C60-13K55 | Boring Tool Finish |  |
|  | WRIST PIN BORE |  |  |
|  | TOOL NUMBER | DESCRIPTION |  |
|  | C60-10497 | Boring Bar |  |
|  | C60-13K38 | Boring Tool Rough |  |
|  | C60-10493 | Boring Tool Finish |  |

GAGING INSTRUCTIONS

| GAGE NUMBER | DESCRIPTION | FREQUENCY |
| --- | --- | --- |
| X-435 & X-436 | Gage Center to Centerline Distance Bend and Twist Parallelism of Bores | 1/50 |
| Sheffield Air Electric | Pin & Crank Bore Size Bore Size range 0.4376/0.4380 | 1/10 |
| NO-C-177 | Bore Finish Checking 16 Micro-Finish Bendix Prof Ilometer | 1/50 |