AMERICOLD MANUFACTURING PROCESS SHEET

| PROCESS DESCRIPTION: MACHINING ALUMINUM CONNECTING ROD : Drill (0.0938) 3/32 Oil Hole | PS #: 2104-001 |
| --- | --- |
| DEPARTMENT: 2104 | REV.: 1 |
| PART NAME: ALUMINUM CONNECTING ROD | DATE: 8-14-95 |
| PART NO.: ALL SERIES ROD | PREPARED BY: P. Brandon |
| MACHINE NAME: GMI DRILL | APPROVED BY: |
| MACHINE NUMBER: 1-0284, 1-0283 | PAGE 1 OF 1 |

- Drill 3/32 inch (0.0938) diameter Oil Hole.

TOOLS REQUIRED

|  | TOOL NUMBER | DESCRIPTION |  |
| --- | --- | --- | --- |
|  | C60-13J18 | Locator Assembly |  |
|  | C60-13K22 | P.12 3/32 I.D. Carbide Drill Bushing |  |
|  | C60-13K44 | #13 Center Drill |  |
|  | C60-13K23 | Erickson #300 Collet |  |
|  | C60-13K24 | Erickson Chuck & Nert for #300 Collet |  |
|  | C60-13J19 | Drill 3/32 inch (0.0938) H.S.S. Fast Spiral |  |

SET UP PARAMETERS

- SET CENTER DRILL TO DEPTH

Head No. 2 depth can be determined by measuring across the width of the countersink.  It should measure 1/8 inches (0.125) ± 0.015.

Measuring Tool:  6 inches machinist scale on Calipers.

- SET DRILLING DEPTH FOR HEAD NO.3.

Set drilling depth for Head No. 3 for a overall depth of 5/8 inches (0.625 inches) deep.  Note set rapid tranel set down mechanism to prevent engaging can/rod during rapid feed.  The rapid tranel feature at all the drills will need adjusting with the initial set up.

- Station four (4) setup for an overall in feed of 7/8 (0.875) inch depth ± 0.015.

- Station five (5) setup for an overall in feed of 1 1/8 (1.125) inch depth ± 0-015

- Station six (6) setup for an overall in feed of 1 3/8 (1.375) inch depth ± 0.015.

- Station seven (7) setup for an overall in feed of 1 1/2 (1.500) inch depth ± 0.015.

SPINDLE SPEED:  6000 RPM FIXED

FEED RATE :  INFINITE VARIABLE 60-6000 mm-min.

Function Gage:  #P.102 Go: 0.0926- No Go: 0.099		Frequency: 1/50 pieces produced