AMERICOLD MANUFACTURING  OPERATOR INSTRUCTIONS

| JOB CLASS: 0651 | MFGOI #: 2408-019 |
| --- | --- |
| OPERATION: Torch Braze | REV.: #3 |
| DEPARTMENT: 2408 | DATE: 10-30-96 |
| LOCATION: White Room | PREPARED BY: R. Hudson |
| MACHINE NAME: New Assembly Line | APPROVED BY: |
| MACHINE NUMBER: 4-0122 | PAGE 1 OF 2 |

START UP:

1. Check parts against Build Up Ticket to insure the correct parts are present for assembly. If the correct parts are not present for assembly, notify Group Leader or Supervisor.

2. Turn on Argon and Oxygen regulators located beside exhaust fan system.

3. Light torch and adjust flame until dark blue portion of flame extends approximately 1/4 to 3/8 inch from tip of torch.

4. Turn on exhaust fan by pressing start button.

5. Flush filters by simultaneously pressing both black buttons on the left rear of unit at waist height.

OPERATOR INSTRUCTIONS:

1. Heat should always be first applied to lower discharge tube. This will pull braze material down into lower tube.

2. Place heat shield between stator and discharge tube.

3. Apply torch flame to discharge tube of lower dome until tube begins to change to a red color.

4. Discharge tube should never be over heated. This will cause discharge tube to be brittle and cause field failure. If Discharge tube is over heated, reject compressor by pushing flag on pallet; this will send the compressor to repair.

5. Apply wire/flux to the tube juncture leaving torch in place.

6. When wire/flux runs smoothly 360° around tube, remove torch.

AMERICOLD MANUFACTURING  OPERATOR INSTRUCTIONS

| CONTINUATION SHEET | MFGOI #: 2408-019 |
| --- | --- |
| OPERATION: Torch Braze | REV.: #3 |
| DEPARTMENT: 2408 | DATE: 10-30-96 |
| MACHINE NUMBER: 4-0122 | PAGE 2 OF 2 |

| OPERATOR INSTRUCTIONS CON’T: |  |  |  |
| --- | --- | --- | --- |
| 7. The braze joint should be completely full 360°. If discharge braze joint is not completely full and braze is not drawn down into lower tube, this can cause a flux glassing effect, or field failure; reject compressor by pushing flag on pallet. |  |  |  |
| 8. Remove heat shield. Periodically clean heat shield. |  |  |  |
| 9. Press pallet cylinder stop control button to index carrousel. |  |  |  |
| PREVENTIVE MAINTENANCE: |  |  |  |
| 1. Maintain a clean and orderly work area. |  |  |  |
| 2. Clean work area at end of shift. |  |  |  |
| CHANGE OVER: NONE |  |  |  |
| OPERATOR CHECKS: NONE |  |  |  |
| PERSONAL PROTECTIVE EQUIPMENT REQUIREMENTS: |  |  |  |
| 1. Tinted Safety Glasses and gloves are required. |  |  |  |
| PARTS LIST: |  |  |  |
|  | PART NAME | PART NUMBER |  |
|  | Heat Shield | C60-15999 |  |
|  | Torch | 066-25120 |  |
|  | Striker | 066-25125 |  |
|  | Flint | 066-25126 |  |
|  | Torch Cleaner | C60-15225 |  |
|  | Torch Tip (Male) | 066-25119 |  |