AMERICOLD MANUFACTURING PROCESS SHEET

| PROCESS DESCRIPTION: ROUGH GRIND O.D. OF PISTON | PS #: 2107-001 |
| --- | --- |
| DEPARTMENT: 2107 | REV.: 1 |
| PART NAME: PISTON | DATE: 7-10-95 |
| PART NO.: ALL PISTONS | PREPARED BY: M. Murphy |
| MACHINE NAME: CINCINNATI TWIN GRIP GRINDER | APPROVED BY: |
| MACHINE NUMBER: 1-0343, 1-0058 | PAGE 1 OF 1 |

ROUGH GRIND OUTER DIAMETER OF PISTON AS FOLLOWS:

|  | FINISH SIZE | ROUGH SIZE | MASTER GAGE # |  |
| --- | --- | --- | --- | --- |
|  | .8119 .8123 | .8146 .8149 | B-587 B-560 |  |
|  | .9494 .9498 | .9521 .9524 | B-588 B-569 |  |
|  | .9994 .9998 | 1.0021 1.0025 | B-589 B-562 |  |
|  | 1.0374 1.0378 | 1.0400 1.0403 | B-590 B-563 |  |
|  | 1.0744 1.0748 | 1.0770 1.0773 | B-591 B-564 |  |

Tools Required:

C60-15155	Grinding Wheel

C60-15A56	Regulating Wheel

Set-up Parameters:

8000-9000	SFPM		Wheel Speed

16-17 		in/min.		Feed Rate